Arduino setup guide: Mehdi Tahari

Smart Desk Guardian · Wiring, software, code and logic

# 1  What the Arduino does

The Arduino is the part of Smart Desk Guardian the user actually touches. It does four jobs:

- Reads the sensors: distance to the screen, room light, the button and the heart (ECG) sensor.

- Decides whether something is wrong: too close, sitting too long, too dark or heart rate too high.

- Warns the user on the spot with the LCD (red/green screen with a message) and the buzzer. This works even with no WiFi.

- Sends everything to Angel's Raspberry Pi over WiFi every 2 seconds, so Adam's app can show it. It also listens to the Pi: when the Pi finds a reading above the user's personal normal, the LCD and buzzer react straight away.

## How the user uses it

| User action | What happens |
| --- | --- |
| Short press on the button | Sitting starts: one beep, the timer starts. Press again: sitting stops (two beeps) and the session is sent to the Pi. |
| Hold the button 2 seconds | A 20-second heart check starts. The LCD turns blue, and the user keeps still with the 3 ECG pads on. |
| Sits closer than 50 cm | LCD turns red: "Too close!" + beep |
| Sits 45 minutes or more | LCD turns red: "Stand up!" + beep |
| Room gets dark | LCD turns red: "Too dark!" + beep |
| Above their personal normal (Pi decides) | LCD red: "Personal alert" + "Take a break!" or "Heart rate high!" for 10 s, double beep |
| Everything fine | LCD green: "All good", with the distance and sitting minutes on line 1 |

## Your plan week by week (from the implementation plan)

| Week | What to do | Done when |
| --- | --- | --- |
| 1 | Install the software (section 2). Agree the messages with Angel and Adam (section 9). | A test sketch uploads |
| 2 | Wire and test the ultrasonic sensor and the button (sections 3 and 4). | Serial monitor demo |
| 3 | Add and test the light and ECG sensors; readings printed as JSON. | Complete sensor node |
| 4 | WiFi + MQTT: readings arrive on Angel's Pi (section 8). | Messages visible in an MQTT client |
| 5 | LCD and buzzer alerts: the full sketch (sections 5 to 7). | Local alerts demo |
| 6-7 | Tune the thresholds and test with the app. | Full system demo |
| 8 | Put everything in the enclosure; long test. | Prototype inspection |

# 2  Software to install

- Download and install Arduino IDE 2 from arduino.cc/en/software.

- Install the board package: Tools → Board → Boards Manager, search megaAVR, and install "Arduino megaAVR Boards". This is needed because the Uno WiFi Rev2 uses a different chip (ATmega4809) from the normal Uno.

- Connect the Arduino with its USB cable. Select Tools → Board → Arduino megaAVR Boards → Arduino Uno WiFi Rev2, and Tools → Port → the COM port that shows "Arduino Uno WiFi Rev2".

- Install 3 libraries: Sketch → Include Library → Manage Libraries, search for each one and click Install:

| Library (search for) | What it is for |
| --- | --- |
| WiFiNINA (by Arduino) | The board's built-in WiFi |
| ArduinoMqttClient (by Arduino) | Sending MQTT messages to the Pi |
| Grove - LCD RGB Backlight (by Seeed Studio) | The LCD screen (gives rgb_lcd.h) |

Test the setup: open File → Examples → 01.Basics → Blink and click Upload (the → arrow). If the small LED on the board blinks, everything is installed correctly.

Optional but recommended: update the WiFi module firmware with Tools → Firmware Updater in Arduino IDE 2. Old firmware can cause WiFi connection problems.

# 3  Wiring

## Safety rules

- Always unplug the USB cable before you plug or unplug anything.

- Put the Grove Base Shield on top of the Arduino. All pins must go in straight.

- Set the small switch on the Base Shield to 5V (the LCD and the ultrasonic sensor need 5 V).

## Where each part goes

| Part | Port | Cable |
| --- | --- | --- |
| Grove button | D2 | Grove cable |
| Grove buzzer | D6 | Grove cable |
| Grove Ultrasonic Ranger | D7 | Grove cable |
| Grove light sensor | A0 | Grove cable |
| ECG sensor (DFRobot SEN0213) | A1 | Grove-to-jumper cable (see below) |
| Grove LCD RGB Backlight | any I2C port | Grove cable |

## The ECG sensor: careful with the cable

The ECG board has a 3-pin DFRobot "Gravity" plug, and Grove cables have 4 pins in a different order. Don't force a Grove cable into it, because you could swap power and ground and damage the sensor. Use a Grove-to-female-jumper cable (or jumper wires) and connect them like this:

| Grove cable wire (port A1) | goes to the ECG board pin | meaning |
| --- | --- | --- |
| Yellow | Signal (marked A or OUT) | the heart signal |
| Red | + (VCC) | 5 V power |
| Black | - (GND) | ground |
| White | not connected | (not used) |

Plug the electrode cable into the ECG board and stick the 3 pads on the body following the labels on the cable (right arm, left arm, right leg / lower body). The DFRobot SEN0213 wiki page has a picture.

The ECG sensor is only for a short check while sitting still, not for medical use. Movement and loose pads give wrong values.

# 4  Test each part on its own first

Before the big program, test every part with a tiny sketch. If something doesn't work later, you will know the wiring is fine. For each one: File → New, paste it, Upload, then open Tools → Serial Monitor (set it to the same baud number as in Serial.begin).

### Test 1: Button and buzzer (D2, D6)

void setup() {

pinMode(2, INPUT);     // button

pinMode(6, OUTPUT);    // buzzer

Serial.begin(9600);

}

void loop() {

int pressed = digitalRead(2);   // 1 = pressed, 0 = not pressed

digitalWrite(6, pressed);       // buzzer on while the button is pressed

Serial.println(pressed);

delay(50);

}

Expected: the buzzer sounds while you press the button, and the monitor shows 1.

### Test 2: Light sensor (A0)

void setup() { Serial.begin(9600); }

void loop() {

Serial.println(analogRead(A0));   // 0 = dark ... 1023 = bright

delay(500);

}

Expected: the number drops when you cover the sensor with your hand. Write down the value in normal room light and with the lights off. You need them in section 7.

### Test 3: Ultrasonic distance (D7)

void setup() { Serial.begin(9600); }

void loop() {

pinMode(7, OUTPUT);                       // send a short sound "ping"

digitalWrite(7, LOW);  delayMicroseconds(2);

digitalWrite(7, HIGH); delayMicroseconds(5);

digitalWrite(7, LOW);

pinMode(7, INPUT);                        // wait for the echo

long echoTime = pulseIn(7, HIGH, 25000);  // in microseconds

Serial.print(echoTime / 29 / 2);          // convert to cm

Serial.println(" cm");

delay(500);

}

How it works: the sensor sends a sound you can't hear and measures how long the echo takes to come back. Sound travels 1 cm in about 29 microseconds, and it goes there and back, so we divide by 29 and then by 2. Expected: move your hand closer and the number goes down. 0 means nothing was detected.

### Test 4: LCD (I2C)

#include <Wire.h>

#include "rgb_lcd.h"

rgb_lcd lcd;

void setup() {

lcd.begin(16, 2);           // 16 characters, 2 lines

lcd.setRGB(0, 255, 0);      // green background (red, green, blue: 0-255)

lcd.print("Hello Mehdi!");

lcd.setCursor(0, 1);        // column 0, line 2

lcd.print("LCD works");

}

void loop() { }

Expected: a green screen with two lines of text. If the colour changes but there is no text, turn the small contrast screw on the back (if your version has one) or check the 5V switch.

### Test 5: ECG signal (A1)

void setup() { Serial.begin(115200); }

void loop() {

Serial.println(analogRead(A1));

delay(5);                    // 200 readings per second

}

Put the pads on, sit still, and open Tools → Serial Plotter (not the monitor) at 115200 baud. Expected: a line with a tall sharp spike at every heartbeat. These spikes are what the program counts. If the line is flat or very messy, check the pads and the cable.

### Test 6: WiFi

#include <WiFiNINA.h>

void setup() {

Serial.begin(9600);

while (!Serial);                                   // wait for the monitor

Serial.println("Connecting...");

WiFi.begin("your-wifi-name", "your-wifi-password");

if (WiFi.status() == WL_CONNECTED) {

Serial.print("Connected! My IP: ");

Serial.println(WiFi.localIP());

} else {

Serial.println("Failed");

}

}

void loop() { }

The Uno WiFi Rev2 only works with 2.4 GHz WiFi with a normal password. A school network that needs a login page or a username (like eduroam) won't work. Use a phone hotspot or the Pi's own WiFi for the demo instead. Use the same network as Angel's Pi.

# 5  The full program

When all 6 tests work, create the real sketch: File → New, save it as sdg_desk, and paste the code below. The same file is also saved in the arduino/sdg_desk folder next to this document.

Before uploading, change the settings in part 1: your WiFi name and password, and the Pi's IP address (Angel finds it on the Pi with the command hostname -I).

/*

Smart Desk Guardian - Arduino Uno WiFi Rev2

Reads the sensors, shows alerts on the LCD and buzzer,

and sends the readings to the Raspberry Pi over WiFi (MQTT).

Short press on the button  = start / stop sitting

Long press (2 s or more)   = start a 20 second ECG heart check

Personal alerts from the Pi (sdg/alerts) are shown straight away

*/

#include <WiFiNINA.h>          // WiFi on the Uno WiFi Rev2

#include <ArduinoMqttClient.h> // MQTT messages

#include <Wire.h>              // I2C, used by the LCD

#include "rgb_lcd.h"           // Grove LCD RGB Backlight

// ---------- 1. SETTINGS: change these ----------

const char WIFI_NAME[]     = "your-wifi-name";

const char WIFI_PASSWORD[] = "your-wifi-password";

const char BROKER[]        = "192.168.1.50";   // the Pi's IP address (ask Angel)

const int  BROKER_PORT     = 1883;

const int TOO_CLOSE_CM   = 50;   // closer than this = too close

const int MAX_SIT_MIN    = 45;   // sitting longer than this = stand up

const int TOO_DARK       = 200;  // light value below this = too dark

const int RESTING_BPM    = 75;   // your own resting heart rate

const int BPM_MARGIN     = 15;   // alert if bpm > RESTING_BPM + BPM_MARGIN

// ---------- 2. PINS (Grove Base Shield ports) ----------

const int BUTTON_PIN     = 2;    // D2

const int BUZZER_PIN     = 6;    // D6

const int ULTRASONIC_PIN = 7;    // D7

const int LIGHT_PIN      = A0;   // A0

const int ECG_PIN        = A1;   // A1

// LCD -> any I2C port

// ---------- 3. VARIABLES (the Arduino's memory) ----------

WiFiClient wifiClient;

MqttClient mqttClient(wifiClient);

rgb_lcd lcd;

bool sitting = false;               // is the user sitting now?

unsigned long sitStart = 0;         // when they sat down (ms)

int distance = 0;                   // cm, 0 = nobody detected

int light = 0;                      // 0 (dark) to 1023 (bright)

int lastBpm = 0;                    // result of the last ECG check

unsigned long ecgDoneAt = 0;        // when the last ECG check ended

bool hadAlert = false;              // was there an alert last time?

unsigned long lastSend = 0;         // last time we sent readings

unsigned long lastReconnect = 0;    // last time we tried to reconnect

String piAlertText = "";            // last personal alert from the Pi

unsigned long piAlertUntil = 0;     // show it on the LCD until this time

bool ecgRunning = false;            // true during an ECG check

bool lastButton = LOW;

unsigned long pressedAt = 0;

// ======================================================

void setup() {

Serial.begin(9600);

pinMode(BUTTON_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

lcd.begin(16, 2);

lcd.setRGB(0, 0, 255);

lcd.print("Smart Desk");

lcd.setCursor(0, 1);

lcd.print("Guardian...");

connectWifiAndMqtt();

}

// ======================================================

void loop() {

checkButton();                    // check the button as often as possible

if (millis() - lastSend >= 2000) { // every 2 seconds:

lastSend = millis();

distance = readDistance();

light = analogRead(LIGHT_PIN);

showAndWarn();                   // LCD + buzzer

sendReadings();                  // to the Pi

}

// If WiFi or MQTT was lost, try again every 30 s (alerts keep working meanwhile)

if (!mqttClient.connected() && millis() - lastReconnect > 30000) {

lastReconnect = millis();

connectWifiAndMqtt();

}

mqttClient.poll();                // keeps the MQTT connection alive

}

// ---------- BUTTON: short press = sitting, long press = ECG ----------

void checkButton() {

bool button = digitalRead(BUTTON_PIN);   // HIGH = pressed

if (button == HIGH && lastButton == LOW) {        // just pressed

pressedAt = millis();

}

if (button == LOW && lastButton == HIGH) {        // just released

unsigned long held = millis() - pressedAt;

if (held >= 2000) {

runEcgCheck();

} else if (held > 50) {                         // ignore "bounces" under 50 ms

toggleSitting();

}

}

lastButton = button;

}

void toggleSitting() {

if (!sitting) {

sitting = true;

sitStart = millis();

publish("sdg/session", "{\"event\": \"start\"}");

beep(100);

} else {

sitting = false;

unsigned long seconds = (millis() - sitStart) / 1000;

publish("sdg/session", "{\"event\": \"stop\", \"duration_s\": " + String(seconds) + "}");

beep(100); delay(100); beep(100);

}

}

// ---------- ULTRASONIC: distance in cm ----------

int readDistance() {

pinMode(ULTRASONIC_PIN, OUTPUT);          // 1. send a short "ping"

digitalWrite(ULTRASONIC_PIN, LOW);

delayMicroseconds(2);

digitalWrite(ULTRASONIC_PIN, HIGH);

delayMicroseconds(5);

digitalWrite(ULTRASONIC_PIN, LOW);

pinMode(ULTRASONIC_PIN, INPUT);           // 2. listen for the echo

long echoTime = pulseIn(ULTRASONIC_PIN, HIGH, 25000);  // microseconds, 0 = no echo

return echoTime / 29 / 2;                 // 3. sound: 29 us per cm, there and back

}

// ---------- SITTING TIME in minutes ----------

int sitMinutes() {

if (!sitting) return 0;

return (millis() - sitStart) / 60000;

}

// ---------- LCD + BUZZER ----------

void showAndWarn() {

// A personal alert from the Pi stays on screen for 10 seconds

if (millis() < piAlertUntil) return;

String warning = "";

if (distance > 0 && distance < TOO_CLOSE_CM) warning = "Too close!";

else if (sitMinutes() >= MAX_SIT_MIN)        warning = "Stand up!";

else if (light < TOO_DARK)                   warning = "Too dark!";

// Line 1: the values

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(String(distance) + "cm  " + (sitting ? String(sitMinutes()) + "min" : "standing"));

// Line 2: warning, or OK

lcd.setCursor(0, 1);

if (warning != "") {

lcd.setRGB(255, 0, 0);          // red

lcd.print(warning);

if (!hadAlert) beep(300);       // beep only when a NEW alert starts

hadAlert = true;

} else {

lcd.setRGB(0, 255, 0);          // green

lcd.print("All good");

hadAlert = false;

}

}

void beep(int ms) {

digitalWrite(BUZZER_PIN, HIGH);

delay(ms);

digitalWrite(BUZZER_PIN, LOW);

}

// ---------- ECG HEART CHECK (20 seconds, user keeps still) ----------

void runEcgCheck() {

ecgRunning = true;

lcd.clear();

lcd.setRGB(0, 0, 255);            // blue

lcd.print("ECG check 20s");

lcd.setCursor(0, 1);

lcd.print("Keep still...");

unsigned long start = millis();

unsigned long lastSample = 0;

unsigned long lastBeat = 0;

unsigned long intervalSum = 0;

int beats = 0;

int minV = 1023, maxV = 0;

while (millis() - start < 20000) {

if (millis() - lastSample >= 5) {               // read 200 times per second

lastSample = millis();

int v = analogRead(ECG_PIN);

if (millis() - start < 3000) {                // first 3 s: learn the signal range

if (v < minV) minV = v;

if (v > maxV) maxV = v;

} else {

int threshold = minV + (maxV - minV) * 7 / 10;   // 70% of the way to the top

if (v > threshold && millis() - lastBeat > 300) { // a heartbeat peak

if (lastBeat > 0) {

intervalSum += millis() - lastBeat;     // time since the previous beat

beats++;

}

lastBeat = millis();

}

}

}

mqttClient.poll();

}

lcd.clear();

if (beats < 5) {                                  // not enough clean beats

lcd.setRGB(255, 120, 0);

lcd.print("ECG failed");

lcd.setCursor(0, 1);

lcd.print("Check pads");

lastBpm = 0;

} else {

lastBpm = 60000UL * beats / intervalSum;        // beats per minute

ecgDoneAt = millis();

lcd.print("Heart: " + String(lastBpm) + " bpm");

lcd.setCursor(0, 1);

if (lastBpm > RESTING_BPM + BPM_MARGIN) {

lcd.setRGB(255, 0, 0);

lcd.print("Above resting!");

beep(300);

} else {

lcd.setRGB(0, 255, 0);

lcd.print("Normal");

}

}

delay(4000);                                      // show the result for 4 s

ecgRunning = false;

}

// ---------- PERSONAL ALERT FROM THE PI (runs as soon as one arrives) ----------

void onPiAlert(int messageSize) {

String text = "";

while (mqttClient.available()) {                  // read the message letter by letter

text += (char)mqttClient.read();

}

Serial.println("Pi alert: " + text);

if (text.indexOf("\"type\": \"heart\"") >= 0) piAlertText = "Heart rate high!";

else                                           piAlertText = "Take a break!";

if (ecgRunning) return;                           // don't disturb an ECG check

piAlertUntil = millis() + 10000;                  // keep it on the LCD for 10 s

lcd.clear();

lcd.setRGB(255, 0, 0);                            // red

lcd.print("Personal alert");

lcd.setCursor(0, 1);

lcd.print(piAlertText);

beep(300); delay(150); beep(300);                 // double beep = from the Pi

}

// ---------- SEND TO THE PI ----------

void sendReadings() {

int bpm = 0;

if (lastBpm > 0 && millis() - ecgDoneAt < 60000) bpm = lastBpm;  // show for 1 minute

String json = "{\"distance\": " + String(distance) +

", \"light\": " + String(light) +

", \"sitting\": " + (sitting ? "true" : "false") +

", \"sit_minutes\": " + String(sitMinutes()) +

", \"bpm\": " + String(bpm) + "}";

publish("sdg/readings", json);

Serial.println(json);             // also show it in the Serial Monitor

}

void publish(const char* topic, const String& text) {

if (!mqttClient.connected()) return;   // offline: skip, alerts still work

mqttClient.beginMessage(topic);

mqttClient.print(text);

mqttClient.endMessage();

}

// ---------- WIFI + MQTT CONNECTION ----------

void connectWifiAndMqtt() {

if (WiFi.status() != WL_CONNECTED) {

Serial.print("Connecting to WiFi... ");

WiFi.begin(WIFI_NAME, WIFI_PASSWORD);

if (WiFi.status() != WL_CONNECTED) {

Serial.println("failed (will retry)");

return;

}

Serial.print("OK, my IP: ");

Serial.println(WiFi.localIP());

}

Serial.print("Connecting to the Pi... ");

if (mqttClient.connect(BROKER, BROKER_PORT)) {

Serial.println("OK");

mqttClient.onMessage(onPiAlert);                // call onPiAlert for each message

mqttClient.subscribe("sdg/alerts");             // listen to the Pi's alerts

} else {

Serial.print("failed, error ");

Serial.println(mqttClient.connectError());

}

}

# 6  How the program works

## The main idea: never wait

loop() runs thousands of times per second. Instead of using delay() (which freezes everything, including the button), we look at the clock with millis(), which gives the milliseconds since the Arduino started:

if (millis() - lastSend >= 2000) {   // has it been 2 seconds since last time?

lastSend = millis();                 // remember "now"

...                                  // do the 2-second work

}

This way the button is checked all the time, and the sensors are read every 2 seconds.

## What happens, step by step

| Part of the code | What it does |
| --- | --- |
| setup() | Runs once: starts the LCD, the buzzer and the button, and connects to WiFi and the Pi. |
| loop() | Runs forever: checks the button, and every 2 s reads the sensors, updates the LCD/buzzer and sends the readings. Reconnects every 30 s if the connection was lost. |
| checkButton() | Remembers when the button was pressed. On release: held 2 s or more = ECG check, otherwise = start/stop sitting. Presses shorter than 50 ms are ignored, because a button "bounces" for a few ms when pressed. |
| toggleSitting() | Switches sitting on/off, saves the start time, and sends start or stop (with the duration) to sdg/session. |
| readDistance() | The ultrasonic measurement from Test 3. |
| onPiAlert() | Runs by itself as soon as a message arrives on sdg/alerts (we told the MQTT library to call it with mqttClient.onMessage(onPiAlert) after connecting). It reads the message letter by letter, picks a short text that fits on the 16-character screen, turns the LCD red and double-beeps. It keeps the alert on screen for 10 s. During an ECG check it stays quiet so it doesn't disturb the measurement. |
| showAndWarn() | Skips its work while a personal alert is on screen. Otherwise it checks the 3 rules in order of importance (too close, sitting too long, too dark). Shows a red warning or a green "All good". Beeps only when a new alert starts, so the buzzer doesn't beep every 2 s. |
| runEcgCheck() | For 20 s: reads the ECG 200 times per second. For the first 3 s it only learns the lowest and highest values. After that, every time the signal goes above 70% of that range, it counts a heartbeat. It ignores anything within 300 ms of the last beat, so one spike isn't counted twice. BPM = 60000 / average time between beats (ms). |
| sendReadings() | Builds the JSON text and sends it to sdg/readings. The heart rate is sent for 1 minute after a check, then 0. |
| publish() | Sends one MQTT message, or does nothing if we are offline. That is why the alerts keep working without WiFi. |

## Example: the heart rate maths

Say the check found 20 beats, and the times between them add up to 16,000 ms. The average time between beats = 16,000 / 20 = 800 ms. In one minute (60,000 ms) that is 60,000 / 800 = 75 beats per minute. In the code: 60000 * beats / intervalSum = 60000 × 20 / 16000 = 75.

# 7  Tune the numbers (calibration)

All the limits are at the top of the program, so you only change them in one place:

| Setting | Start | How to choose |
| --- | --- | --- |
| TOO_CLOSE_CM | 50 | From the project goal. Check with a ruler that the sensor reading matches. |
| MAX_SIT_MIN | 45 | From the project goal. For the demo, set it to 1 so you don't wait 45 minutes! |
| TOO_DARK | 200 | Between your "lights on" and "lights off" values from Test 2. |
| RESTING_BPM | 75 | Do an ECG check while relaxed and use that value. |
| BPM_MARGIN | 15 | How far above resting counts as "high". |

Where to put the ultrasonic sensor: on top of or under the screen, pointing at the user's face or chest. If it is aimed at the desk, it measures the desk.

# 8  Test with the Pi

- Make sure Angel's Pi is on and Mosquitto is running (his Step 3).

- Put the Pi's IP address in BROKER, upload, and open the Serial Monitor at 9600. You should see Connecting to the Pi... OK, then a JSON line every 2 seconds.

- On your laptop, open MQTT Explorer, connect to the Pi's IP on port 1883, and watch the sdg topics. Your readings appear every 2 s, and a sdg/session message appears when you press the button.

- Press the button, wait, press again: Angel's history page should show the new session.

- Test the personal alerts: ask Angel to load his demo data (his Step 6) and send his fake reading. Your LCD should turn red with "Personal alert / Take a break!" and double-beep within a second.

# 9  The messages (shared with Angel and Adam)

There are three kinds of messages: the Arduino sends the first two, and the Pi sends the third. Think of a topic as the name of a channel. Whoever is listening to that channel receives the message.

| Topic | What is in it |
| --- | --- |
| sdg/readings | Sent every 2 seconds. All the current values from the desk. |
| sdg/session | Sent when the user presses the button: sitting started, or sitting stopped (and for how long). |
| sdg/alerts | Sent by the Pi when a reading is above the user's personal normal (see the personal limits step). |

Example messages (this format is called JSON, which is simply names with values):

sdg/readings   {"distance": 62, "light": 540, "sitting": true, "sit_minutes": 22, "bpm": 0}

sdg/session    {"event": "start"}

sdg/session    {"event": "stop", "duration_s": 2710}

sdg/alerts     {"type": "sitting", "msg": "You have been sitting for 48 min. Your normal is 47 min. Time for a break!"}

- distance: how far the user is from the screen, in cm (too close if under 50)

- light: how bright the room is, from 0 (dark) to 1023 (bright). Too dark if under 200.

- sitting and sit_minutes: whether the user is sitting and for how many minutes (too long from 45)

- bpm: heart rate from the last ECG check. It is sent for 1 minute after the check, otherwise 0.

- type and msg (alerts only): the kind of warning (sitting or heart) and the text to show the user

- duration_s: how long the user sat, in seconds (2710 s = 45 min 10 s)

Mehdi's Arduino and Angel's Pi send exactly this format, so please don't change it without telling the team.

The Arduino sends the first two (sendReadings() and toggleSitting() already use exactly this format) and listens to the third (onPiAlert()).

# 10  If something goes wrong

| Problem | Try this |
| --- | --- |
| Upload fails / no port | Different USB cable (some only charge). Check the board is "Arduino Uno WiFi Rev2". |
| rgb_lcd.h: No such file | Install the "Grove - LCD RGB Backlight" library. |
| Distance always 0 | Wrong port (must be D7), or nothing in front of the sensor within about 4 m. |
| LCD stays blank | 5V switch on the shield, I2C port, cable pushed in fully. |
| "ECG failed - Check pads" | Pads not stuck well, moving or talking during the check, or the cable is wired wrong (Test 5). |
| WiFi "failed" | 2.4 GHz network? Name and password exactly right (capital letters)? Update the firmware. |
| Pi connection "failed, error -2" | Wrong IP, the Pi is off, or it is on a different network. |
| Everything resets randomly | Weak USB port: use a powered USB hub or a 9 V adapter on the barrel jack. |

## Optional extra: auto-stop when the user leaves

If the user forgets to press the button when leaving, the ultrasonic sensor can notice. Add this inside the 2-second part of loop():

static unsigned long awaySince = 0;

if (sitting && (distance == 0 || distance > 150)) {   // nobody at the desk

if (awaySince == 0) awaySince = millis();

if (millis() - awaySince > 300000) toggleSitting();  // gone for 5 minutes: stop

} else {

awaySince = 0;                                        // someone is there again

}