Raspberry Pi tasks: Angel Velazquez

Smart Desk Guardian · Beginner guide · prepared by Mehdi Tahari (project manager)

# 1  Your job in one sentence

You set up the Raspberry Pi as the "post office" of our project: the Arduino sends its messages to the Pi, and Adam's app picks them up from the Pi.

The Pi is a small computer. You only need to make it do three things:

- Run an operating system (like Windows on a laptop, but for the Pi).

- Run Mosquitto, a free program that receives messages and passes them on. This is called an MQTT broker.

- Remember the sitting history with a short Python program (about 30 lines, all given below), so the app can show past sessions.

Not your part: the sensors, the button, the LCD and the buzzer. Mehdi does everything on the Arduino.

Mehdi is always available to help you. Angel, you don't have to figure everything out alone. If a step doesn't work, you don't understand something, or you just want me to check your work, message me any time. We can also sit together and do it step by step. We are all learning, and asking early is the fastest way.

## The big picture

Arduino  --- sends messages --->  Raspberry Pi (Mosquitto)  --- passes them on --->  Adam's app

|

history.py saves finished sessions

and gives the app the list

## Your plan week by week

| Week | What to do | You know it works when |
| --- | --- | --- |
| 1 | Read this document. The Pi, SD card and power supply: we already got those. | You understand the big picture |
| 2 | Step 1 and Step 2: install the OS and connect from your laptop. | You can type commands on the Pi from your laptop |
| 3 | Step 3: install Mosquitto and test it. | A test message arrives |
| 4 | Test with Mehdi's Arduino. | Arduino messages arrive on the Pi |
| 5-6 | Step 4 and Step 5: save the history and start it automatically. | The history page shows sessions |
| 7 | Test everything together with Mehdi and Adam. | The full demo works |
| 8-9 | Fix problems; optional extras if there is time; help with the demo. | Final demo |

# 2  The messages we use

The Arduino sends two kinds of messages. Think of a topic as the name of a channel. Whoever is listening to that channel receives the message.

| Topic | What is in it |
| --- | --- |
| sdg/readings | Sent every 2 seconds. All the current values from the desk. |
| sdg/session | Sent when the user presses the button: sitting started, or sitting stopped (and for how long). |

Example messages (this format is called JSON, which is simply names with values):

sdg/readings   {"distance": 62, "light": 540, "sitting": true, "sit_minutes": 22, "bpm": 0}

sdg/session    {"event": "start"}

sdg/session    {"event": "stop", "duration_s": 2710}

- distance: how far the user is from the screen, in cm (too close if under 50)

- light: how bright the room is, from 0 (dark) to 1023 (bright). Too dark if under 200.

- sitting and sit_minutes: whether the user is sitting and for how many minutes (too long from 45)

- bpm: heart rate. It is 0 when no ECG check is running.

- duration_s: how long the user sat, in seconds (2710 s = 45 min 10 s)

Mehdi's Arduino sends exactly this format, so please don't change it without telling the team.

# 3  Step 1: Put the operating system on the SD card

What you need: the Pi, the microSD card and the power supply (we already got those), your laptop, and the WiFi name and password.

No USB microSD adapter? Can I use an Android phone instead? Not for this step. When a phone is plugged into a laptop, it only shares its files. Raspberry Pi Imager needs to write to the card directly, and a phone doesn't allow that, so the card won't even appear in Imager. What works: a laptop SD slot with a microSD-to-SD adapter (often in the card's box), borrowing a reader at school, or buying one for about 5 to 10 euros.

We use a free program called Raspberry Pi Imager. It erases the card, installs the OS and sets everything up in one go.

- Download Raspberry Pi Imager from raspberrypi.com/software and install it.

- Put the microSD card in your laptop and open Imager.

- Click Choose Device and pick your Pi model.

- Click Choose OS → Raspberry Pi OS (other) → Raspberry Pi OS Lite (64-bit). "Lite" means no desktop and no mouse: we control it from the laptop, which is easier.

- Click Choose Storage and pick the SD card. Be careful: everything on it will be erased.

- Click Next → Edit Settings and fill in the table below. These settings save you from needing a screen or keyboard for the Pi.

- Click Save → Yes. Wait until it says it is finished (about 5 minutes).

- Put the card in the Pi and plug in the power. Wait 2 minutes.

| Setting | What to type |
| --- | --- |
| Hostname | sdg-pi (the name of the Pi on the network) |
| Username | sdg |
| Password | Choose one and write it down |
| Wireless LAN | The WiFi name and password (the laptop and the Arduino will use the same WiFi) |
| Services tab | Tick Enable SSH (this lets you control the Pi from your laptop) |

# 4  Step 2: Control the Pi from your laptop

SSH lets you type commands on the Pi from your laptop's terminal, as if you were sitting at the Pi.

On Windows, open PowerShell (search for it in the Start menu) and type:

ssh sdg@sdg-pi.local

What this means: "connect as user sdg to the computer called sdg-pi". The first time, type yes, then your password. While you type the password nothing appears on screen. That is normal.

When you see sdg@sdg-pi:~ $, you are on the Pi. Update it first. sudo means "run as administrator", and apt is the Pi's app store:

sudo apt update

sudo apt upgrade -y

If the Pi is not found: wait another minute, check that the laptop is on the same WiFi, or ask Mehdi to help find the Pi's IP address (it looks like 192.168.1.23) and use ssh sdg@192.168.1.23 instead.

# 5  Step 3: Install Mosquitto (the post office)

Install it with one command:

sudo apt install -y mosquitto mosquitto-clients

By default, Mosquitto only accepts messages from the Pi itself. We need to tell it to also accept messages from the Arduino and the app. Open a new settings file with the nano text editor:

sudo nano /etc/mosquitto/conf.d/sdg.conf

Type these lines:

listener 1883

listener 9001

protocol websockets

allow_anonymous true

What each line means:

- listener 1883: accept normal MQTT messages on "door" (port) 1883. The Arduino uses this one.

- listener 9001 + protocol websockets: also accept on port 9001 in the format web browsers use. Adam's app uses this one.

- allow_anonymous true: no password needed. This keeps it simple inside our own WiFi. Adding a password is an optional extra later.

Save and close nano: press Ctrl+O, Enter, then Ctrl+X. Restart Mosquitto so it reads the new settings:

sudo systemctl restart mosquitto

## Test it: send yourself a message

Open two PowerShell windows and SSH into the Pi in both. In window 1, listen to everything that starts with sdg/ (the # means "everything"):

mosquitto_sub -h localhost -t "sdg/#" -v

In window 2, send a fake message, pretending to be the Arduino:

mosquitto_pub -h localhost -t sdg/readings -m '{"distance": 42, "light": 500, "sitting": true, "sit_minutes": 3, "bpm": 0}'

Result: the message appears in window 1. Your post office works!

Tip: install MQTT Explorer (free) on your laptop and connect it to sdg-pi.local, port 1883. You will see every message arrive, which is very useful when testing with Mehdi's Arduino. Adam can also use your fake-message command to test his app.

# 6  Step 4: Save the sitting history

Mosquitto only passes messages on. It doesn't remember them. So we write a small Python program that:

- listens to sdg/session, and each time a sitting session stops, saves it in a small database file (sdg.db, using SQLite, which is built into Python)

- shows the last 20 sessions at a web address, so Adam's app can read them: http://sdg-pi.local:5000/history

## Prepare a folder

These commands create a folder, make a private Python environment in it (a "venv" keeps our libraries separate from the system), and install the 3 libraries we need:

mkdir ~/sdg

cd ~/sdg

python3 -m venv venv

venv/bin/pip install paho-mqtt flask flask-cors

- paho-mqtt: lets Python receive MQTT messages

- flask: lets Python show a web page

- flask-cors: allows Adam's app (in a browser) to read that page

## Write the program

Run nano history.py and type this (or copy it from our Git repo):

import json

import sqlite3

import paho.mqtt.client as mqtt

from flask import Flask, jsonify

from flask_cors import CORS

# 1. Open (or create) the database and a table for sessions

db = sqlite3.connect("sdg.db", check_same_thread=False)

db.execute("CREATE TABLE IF NOT EXISTS sessions (ended TEXT, minutes INTEGER)")

# 2. What to do when a message arrives on sdg/session

def on_message(client, userdata, msg):

data = json.loads(msg.payload)                 # text -> Python dictionary

if data["event"] == "stop":

minutes = data["duration_s"] // 60

db.execute("INSERT INTO sessions VALUES (datetime('now', 'localtime'), ?)", (minutes,))

db.commit()                                # really write it to the file

print("Saved a session of", minutes, "minutes")

# 3. Connect to Mosquitto and start listening in the background

client = mqtt.Client(mqtt.CallbackAPIVersion.VERSION2)

client.on_message = on_message

client.connect("localhost", 1883)

client.subscribe("sdg/session")

client.loop_start()

# 4. A small web page that returns the last 20 sessions

app = Flask(__name__)

CORS(app)

@app.get("/history")

def history():

rows = db.execute("SELECT ended, minutes FROM sessions ORDER BY rowid DESC LIMIT 20").fetchall()

return jsonify([{"ended": r[0], "minutes": r[1]} for r in rows])

app.run(host="0.0.0.0", port=5000)

How it works, part by part:

- Part 1 creates a table, which is like a spreadsheet with two columns: when the session ended, and how many minutes it lasted.

- Part 2 runs automatically for every message. If the message says "stop", it adds a row to the table.

- Part 3 connects to Mosquitto on the same Pi (localhost) and listens to the sdg/session topic.

- Part 4 creates the web address /history. When someone opens it, it reads the table and sends it back as JSON.

## Run and test it

venv/bin/python history.py

Leave it running. In a second PowerShell window on the Pi, pretend a 45-minute session just ended:

mosquitto_pub -h localhost -t sdg/session -m '{"event": "stop", "duration_s": 2710}'

The first window prints Saved a session of 45 minutes. Now open http://sdg-pi.local:5000/history in your laptop's browser. You should see:

[{"ended": "2026-11-12 10:31:05", "minutes": 45}]

That is exactly what Adam's app will read. Stop the program with Ctrl+C.

# 7  Step 5: Start everything automatically

Mosquitto already starts by itself when the Pi turns on. For history.py, we add one line to cron, the Pi's built-in scheduler:

crontab -e

(If it asks which editor to use, choose 1 for nano.) Go to the bottom of the file and add this line:

@reboot sleep 20 && cd /home/sdg/sdg && venv/bin/python history.py >> log.txt 2>&1

What this means: "when the Pi boots, wait 20 seconds, go to our folder, start history.py and write its messages into log.txt". Save with Ctrl+O, Enter, Ctrl+X.

Test: type sudo reboot, wait a minute, and open the history address in your browser again. If it works, you are done with the main part.

# 8  Optional extras (only if there is time)

- User accounts / login: a users table and a login page, so the history belongs to a specific person. Ask Mehdi before starting.

- RFID login (that was an idea from Ari): the user taps a card to identify themselves, instead of typing a username and password.

- A password on Mosquitto, instead of allow_anonymous true.

- A Pi WiFi hotspot, in case the school WiFi blocks our messages on demo day.

# 9  Checklist

| Check | Done? |
| --- | --- |
| I can SSH into the Pi from my laptop |  |
| A test message arrives with mosquitto_sub |  |
| Messages from Mehdi's Arduino arrive |  |
| history.py saves a session and /history shows it |  |
| After a reboot, everything works without touching anything |  |
| My files (history.py and the Mosquitto settings) are copied into the pi/ folder in our Git repo |  |

## If something goes wrong

| Problem | Try this |
| --- | --- |
| sdg-pi.local not found | Same WiFi? Wait a minute. Use the IP address instead. |
| Mosquitto doesn't start | sudo systemctl status mosquitto shows the error. Usually a typing mistake in sdg.conf. |
| No module named flask | You ran python instead of venv/bin/python. |
| History page doesn't open | Is history.py running? Look at log.txt with cat ~/sdg/log.txt. |

Mehdi is always available to help you. Angel, you don't have to figure everything out alone. If a step doesn't work, you don't understand something, or you just want me to check your work, message me any time. We can also sit together and do it step by step. We are all learning, and asking early is the fastest way.